# 室内定位技术方案

版本：v1.0
日期：2026-04-23
作者：AI助手

# 一、概述

室内定位技术是一种在GPS等室外定位系统无法覆盖的室内环境中确定人员或物体位置的技术。随着智慧建筑、智慧仓储、人员管理等需求的增长，室内定位技术越来越受到关注。

本方案采用蓝牙低功耗（BLE）技术结合惯性导航（PDR）实现室内定位，具有成本低、功耗低、易部署等优点。

# 二、硬件准备清单

## 2.1 核心硬件

| 硬件类型 | 推荐型号/规格 | 数量 | 预估单价 | 备注 |
| --- | --- | --- | --- | --- |
| BLE信标（定位基站） | nRF51822 / ESP32 | 20-30个 | ¥30-50/个 | 支持BLE 4.0/5.0广播 |
| 测试手机 | 小米/华为/OPPO各1台 | 2-3台 | ¥0 | 需支持BLE+IMU |
| 边缘网关（可选） | 树莓派4B 4G | 1台 | ¥450-600 | 或NUC |
| SD卡+电源 | 16G+5V3A | 1套 | ¥100 | 树莓派配件 |

## 2.2 快速启动方案（省钱版）

如需快速验证，可采用纯手机模拟方案：

• 信标：使用旧手机作为BLE信标（安装nRF Connect或Beacon Simulator App）
• 数量：5-10台旧手机
• 成本：¥0（利用现有设备）
• 优点：零成本、快速验证
• 缺点：手机作为信标不稳定（电量/后台被杀）

## 2.3 推荐购买清单

| 方案 | 配置 | 预算 |
| --- | --- | --- |
| 方案A：经济型 | BLE信标×10个 + 杜邦线/电池盒 | ¥300 |
| 方案B：标准型 | BLE信标×20个 + 树莓派4B | ¥600 |
| 方案C：完整型 | BLE信标×30个 + NUC小主机 | ¥1500 |

⚠️ 建议：从方案A开始，验证后按需升级

# 三、算法实现方案

## 3.1 整体架构

定位算法采用多层融合架构，包括：
1. BLE定位层：基于RSSI的三边定位或指纹定位
2. PDR层：行人航位推算（步态检测+航向估计+步长估计）
3. 融合层：卡尔曼滤波自适应融合BLE和PDR
4. 地图约束层：穿墙检测和位置修正

## 3.2 核心代码实现

### 3.2.1 定位服务主模块（positioning_service.py）

"""
室内定位服务 v1.0
核心模块：BLE定位 + PDR + 融合 + 地图约束
"""

import numpy as np
from dataclasses import dataclass
from typing import List, Optional, Tuple
from enum import Enum
import time


@dataclass
class Position:
    """定位结果"""
    x: float           # X坐标（米）
    y: float           # Y坐标（米）
    floor: int         # 楼层
    confidence: float  # 置信度 [0, 1]
    mode: str          # 定位模式
    timestamp: float   # 时间戳


@dataclass
class BeaconSignal:
    """信标信号"""
    beacon_id: str
    rssi: float        # 信号强度 dBm
    tx_power: float    # 标称发射功率 dBm
    timestamp: float


@dataclass
class IMUData:
    """IMU数据"""
    acc_x: float
    acc_y: float
    acc_z: float
    gyro_x: float
    gyro_y: float
    gyro_z: float
    timestamp: float


class PositioningService:
    """定位服务主类"""
    
    def __init__(self, building_map: 'BuildingMap'):
        self.map = building_map
        self.ble_positioning = BLEPositioning(building_map.beacons)
        self.pdr = PDRModule()
        self.fusion = KalmanFilterFusion()
        self.map_matcher = MapMatcher(building_map)
        self.last_position: Optional[Position] = None
        self.mode = 'NORMAL'
        self.confidence = 1.0
        self.min_beacons = 3
        self.low_confidence_threshold = 0.5
    
    def update(self, beacon_signals: List[BeaconSignal], 
               imu_data: IMUData) -> Position:
        """定位更新（主入口）"""
        # Step 1: BLE定位
        if len(beacon_signals) >= self.min_beacons:
            ble_position = self.ble_positioning.locate(beacon_signals)
            ble_confidence = self._calculate_ble_confidence(beacon_signals)
        else:
            ble_position = None
            ble_confidence = 0.0
        
        # Step 2: PDR定位
        pdr_delta = self.pdr.update(imu_data, self.last_position)
        pdr_position = self._apply_pdr_delta(pdr_delta)
        pdr_confidence = self.pdr.get_confidence()
        
        # Step 3: 融合
        if ble_position:
            fused = self.fusion.fuse(ble_position, ble_confidence,
                                     pdr_position, pdr_confidence)
            self.mode = 'BLE_PDR'
        else:
            fused = pdr_position
            self.mode = 'PDR_ONLY'
            self.confidence = pdr_confidence
        
        # Step 4: 地图约束
        if not self.map_matcher.is_valid(fused):
            fused = self.map_matcher.snap_to_wall(fused)
            self.confidence *= 0.7
        
        # 保存状态
        self.last_position = Position(
            x=fused[0], y=fused[1],
            floor=self.last_position.floor if self.last_position else 1,
            confidence=self.confidence,
            mode=self.mode,
            timestamp=time.time()
        )
        return self.last_position
    
    def _apply_pdr_delta(self, delta) -> Tuple[float, float]:
        if self.last_position:
            return (self.last_position.x + delta[0],
                    self.last_position.y + delta[1])
        return (delta[0], delta[1])
    
    def _calculate_ble_confidence(self, signals: List[BeaconSignal]) -> float:
        if not signals:
            return 0.0
        base = 0.7
        if len(signals) >= 5:
            base += 0.15
        elif len(signals) == 3:
            base -= 0.1
        rssi_values = [s.rssi for s in signals]
        std = np.std(rssi_values)
        stability = max(0, 1 - std / 10)
        base *= (0.5 + 0.5 * stability)
        return min(base, 1.0)
    
    def set_initial_position(self, x: float, y: float, floor: int):
        self.last_position = Position(x=x, y=y, floor=floor,
                                        confidence=1.0, mode='INITIAL',
                                        timestamp=time.time())
        self.pdr.reset()

### 3.2.2 BLE定位模块

class BLEPositioning:
    """BLE定位模块 - 三边定位"""
    
    def __init__(self, beacons: dict):
        """
        Args:
            beacons: 信标字典 {beacon_id: (x, y, tx_power)}
        """
        self.beacons = beacons
        self.mode = 'TRILATERATION'
    
    def locate(self, signals: List[BeaconSignal]) -> Optional[Tuple[float, float]]:
        """定位入口"""
        if self.mode == 'TRILATERATION':
            return self._trilateration(signals)
        return None
    
    def _trilateration(self, signals: List[BeaconSignal]) -> Optional[Tuple[float, float]]:
        """
        三边定位算法
        
        RSSI = TxPower - 10 * n * log10(d)
        d = 10 ^ ((TxPower - RSSI) / (10 * n))
        """
        valid_signals = [s for s in signals if s.beacon_id in self.beacons]
        if len(valid_signals) < 3:
            return None
        
        # 计算距离
        n = 2.0  # 环境因子（室内2-4）
        positions, distances = [], []
        
        for signal in valid_signals:
            beacon_pos = self.beacons[signal.beacon_id]
            d = 10 ** ((signal.tx_power - signal.rssi) / (10 * n))
            positions.append(beacon_pos[:2])
            distances.append(d)
        
        try:
            return self._least_squares(positions, distances)
        except:
            return None
    
    def _least_squares(self, positions: List, distances: List) -> Tuple[float, float]:
        """最小二乘法三角定位"""
        A, b = [], []
        for i in range(1, len(positions)):
            x1, y1 = positions[0]
            xi, yi = positions[i]
            ri, r1 = distances[i], distances[0]
            A.append([2 * (xi - x1), 2 * (yi - y1)])
            b.append([xi**2 + yi**2 - ri**2 - (x1**2 + y1**2 - r1**2)])
        
        A = np.array(A)
        b = np.array(b)
        result = np.linalg.lstsq(A, b, rcond=None)[0]
        return (result[0], result[1])

### 3.2.3 PDR行人航位推算模块

class PDRModule:
    """PDR行人航位推算"""
    
    def __init__(self):
        self.step_length = 0.7  # 默认步长（米）
        self.heading = 0.0     # 朝向角（弧度）
        self.step_threshold = 1.2  # 加速度峰值阈值
        self.step_count = 0
        self.last_step_time = 0
        self.last_acc_magnitude = 0
        self.bias_x = 0.0
        self.bias_y = 0.0
    
    def update(self, imu_data: IMUData, 
               last_position: Optional[Position]) -> Tuple[float, float]:
        """PDR更新，返回位移增量"""
        acc_mag = np.sqrt(imu_data.acc_x**2 + imu_data.acc_y**2 + imu_data.acc_z**2)
        is_step = self._detect_step(acc_mag, imu_data.timestamp)
        
        if is_step:
            self.step_count += 1
            self._update_heading(imu_data)
            delta_x = self.step_length * np.sin(self.heading) - self.bias_x
            delta_y = self.step_length * np.cos(self.heading) - self.bias_y
            return (delta_x, delta_y)
        return (0.0, 0.0)
    
    def _detect_step(self, acc_magnitude: float, timestamp: float) -> bool:
        """步态检测（峰值检测法）"""
        if timestamp - self.last_step_time < 200:
            return False
        if acc_magnitude > self.step_threshold and self.last_acc_magnitude < acc_magnitude:
            self.last_step_time = timestamp
            return True
        self.last_acc_magnitude = acc_magnitude
        return False
    
    def _update_heading(self, imu_data: IMUData):
        """更新航向角（使用陀螺仪积分）"""
        self.heading += imu_data.gyro_z * 0.02
        self.heading = self.heading % (2 * np.pi)
    
    def get_confidence(self) -> float:
        """获取PDR置信度（基于步数递减）"""
        decay = 0.98 ** min(self.step_count, 50)
        return max(0.3, decay)
    
    def calibrate(self, error_x: float, error_y: float):
        """校准偏差"""
        self.bias_x += error_x * 0.5
        self.bias_y += error_y * 0.5
    
    def reset(self):
        """重置"""
        self.step_count = 0
        self.heading = 0.0
        self.bias_x = 0.0
        self.bias_y = 0.0

### 3.2.4 卡尔曼滤波融合模块

class KalmanFilterFusion:
    """卡尔曼滤波融合"""
    
    def __init__(self):
        self.x = np.array([0.0, 0.0, 0.0, 0.0])  # [x, y, vx, vy]
        self.P = np.eye(4) * 100  # 协方差矩阵
        self.Q = np.diag([0.01, 0.01, 0.01, 0.01])  # 过程噪声
        self.R_ble = 1.0
        self.R_pdr = 5.0
    
    def fuse(self, ble_pos: Optional[Tuple], ble_conf: float,
             pdr_pos: Tuple, pdr_conf: float) -> Tuple[float, float]:
        """融合BLE和PDR定位结果"""
        self.R_ble = 10.0 / (ble_conf + 0.1)
        self.R_pdr = 10.0 / (pdr_conf + 0.1)
        
        if ble_pos:
            z = np.array([ble_pos[0], ble_pos[1]])
            H = np.array([[1, 0, 0, 0], [0, 1, 0, 0]])
            R = np.eye(2) * self.R_ble
            self._update(z, H, R)
        
        self._predict()
        return (self.x[0], self.x[1])
    
    def _predict(self):
        """预测步骤"""
        F = np.array([[1, 0, 1, 0], [0, 1, 0, 1], [0, 0, 1, 0], [0, 0, 0, 1]])
        self.x = F @ self.x
        self.P = F @ self.P @ F.T + self.Q
    
    def _update(self, z: np.ndarray, H: np.ndarray, R: np.ndarray):
        """更新步骤"""
        y = z - H @ self.x
        S = H @ self.P @ H.T + R
        K = self.P @ H.T @ np.linalg.inv(S)
        self.x = self.x + K @ y
        I = np.eye(len(self.x))
        self.P = (I - K @ H) @ self.P

### 3.2.5 地图模块

class MapMatcher:
    """地图匹配"""
    
    def __init__(self, building_map: 'BuildingMap'):
        self.map = building_map
    
    def is_valid(self, position: Tuple[float, float]) -> bool:
        """检查位置是否在有效区域内"""
        x, y = position
        if x < 0 or x > self.map.width or y < 0 or y > self.map.height:
            return False
        for obstacle in self.map.obstacles:
            if obstacle.contains(x, y):
                return False
        return True
    
    def snap_to_wall(self, position: Tuple[float, float]) -> Tuple[float, float]:
        """将穿墙位置修正到最近的有效位置"""
        x, y = position
        for obstacle in self.map.obstacles:
            if obstacle.contains(x, y):
                edges = obstacle.get_edges()
                min_dist = float('inf')
                best_point = position
                for edge in edges:
                    closest = self._closest_point_on_segment((x, y), edge[0], edge[1])
                    dist = np.sqrt((closest[0] - x)**2 + (closest[1] - y)**2)
                    if dist < min_dist:
                        min_dist = dist
                        best_point = closest
                return best_point
        return (max(0, min(x, self.map.width)), max(0, min(y, self.map.height)))
    
    def _closest_point_on_segment(self, p: Tuple, a: Tuple, b: Tuple) -> Tuple:
        """求点到线段的最短点"""
        ax, ay = a
        bx, by = b
        px, py = p
        ab = (bx - ax, by - ay)
        ap = (px - ax, py - ay)
        t = max(0, min(1, (ap[0]*ab[0] + ap[1]*ab[1]) / (ab[0]**2 + ab[1]**2 + 1e-10)))
        return (ax + t * ab[0], ay + t * ab[1])


@dataclass
class BuildingMap:
    """建筑地图"""
    width: float
    height: float
    beacons: dict
    obstacles: list
    floors: int = 1

# 四、测试连接方案

## 4.1 开发环境准备

1. Python环境（3.9+）
   • conda create -n indoor_pos python=3.9
   • conda activate indoor_pos
   • pip install numpy matplotlib bleak asyncio

2. Android开发环境（可选）
   • Flutter SDK
   • flutter_blue_plus 插件

3. 硬件连接
   • 树莓派：SSH连接 + Bluetooth配对
   • 或使用USB连接手机进行调试

## 4.2 信标部署指南

部署原则：
• 均匀分布：信标间距3-5m为宜
• 高度一致：距地面2-3m
• 避免遮挡：远离金属/大面积水体
• 边角覆盖：确保边缘区域有信标

10m × 10m 测试区域部署方案：
• 部署5个信标（B1-B5），均匀分布
• 记录每个信标的精确位置（误差<10cm）

## 4.3 测试流程

| 阶段 | 时间 | 测试内容 |
| --- | --- | --- |
| 阶段1：基础测试 | 第1-2周 | 信标信号测试、定位精度测试 |
| 阶段2：PDR测试 | 第3周 | 步态检测验证、PDR累积误差测试 |
| 阶段3：融合测试 | 第4周 | BLE+PDR融合、地图约束测试 |
| 阶段4：实地演练 | 第5-6周 | 连续轨迹测试、极限条件测试 |

## 4.4 测试数据记录模板

{
    "test_name": "定位精度测试",
    "test_date": "2026-04-23",
    "environment": {
        "location": "实验室201",
        "area": "10m x 10m",
        "beacon_count": 5,
        "obstacle_count": 2
    },
    "test_points": [
        {
            "point_id": "P1",
            "ground_truth": {"x": 2.0, "y": 2.0},
            "measurements": [
                {
                    "timestamp": "2026-04-23T10:00:00",
                    "beacon_signals": [
                        {"id": "B1", "rssi": -65},
                        {"id": "B2", "rssi": -72},
                        {"id": "B3", "rssi": -68}
                    ],
                    "estimated_position": {"x": 2.1, "y": 1.9},
                    "confidence": 0.85
                }
            ]
        }
    ],
    "results": {
        "mean_error": 1.2,
        "median_error": 1.0,
        "p95_error": 2.5,
        "accuracy_2m": 85.0,
        "accuracy_3m": 95.0
    }
}

# 五、下一步行动计划

| 时间 | 任务 | 状态 |
| --- | --- | --- |
| 今日 | 准备开发环境（Python 3.9+） | □ 待办 |
| 今日 | 安装依赖：pip install numpy matplotlib bleak | □ 待办 |
| 本周 | 部署信标或准备Beacon模拟App | □ 待办 |
| 本周 | 运行代码测试BLE扫描 | □ 待办 |
| 本周 | 实现三边定位算法 | □ 待办 |
| 下周 | 实现PDR模块 | □ 待办 |
| 下周 | 实现融合定位 | □ 待办 |
| 下周 | 完成基础测试报告 | □ 待办 |